Urban Expansion Monitoring: Lulu Mall Project, Trivandrum (2017–2024)

Post Applied: Imagery Analyst

Prepared by: Ajin P Asok

Date: December 11, 2025

# Introduction & Objectives

This project analyzes urban sprawl in Trivandrum, specifically monitoring the construction of the Lulu Mall complex and surrounding infrastructure. The analysis spans from January 2017 (pre-construction) to January 2024 (post-construction/operational), utilizing a multi-sensor approach with Sentinel-1 (SAR) and Sentinel-2 (Optical) imagery.

- Data Sets Used

The analysis relied on open-source Earth Observation data ingested via the Microsoft Planetary Computer STAC API:

- Sentinel-2 Level-2A (Optical): Bottom-of-Atmosphere (BOA) reflectance.

  - Resolution: 10m (Visible/NIR bands), 20m (SWIR bands).

  - Selection Criteria: Cloud cover < 10%, dry season acquisition (Jan–May) to minimize phenological variance.

- Sentinel-1 (SAR): C-band Synthetic Aperture Radar.

  - Product: Radiometric Terrain Corrected (RTC).

  - Polarization: VV (Vertical-Vertical) and VH (Vertical-Horizontal).

  - Mode: Interferometric Wide (IW).

3. Methods & Results

A. Optical Analysis: NDBI Change Detection

- Method: Calculated the Normalized Difference Built-up Index (NDBI) using SWIR (B11) and NIR (B08) bands.

  - NDBI = (SWIR - NIR) / (SWIR + NIR)

- Why this method: Unlike NDVI (which only detects vegetation loss), NDBI specifically highlights materials with high SWIR reflectance, such as concrete and asphalt.

- Results: The difference map (NDBI_2024 - NDBI_2017) successfully isolated the newly constructed building surrounding the mall. The mall building footprint was already there.

  - Pros: Easy to interpret; distinguishes built-up land from bare soil better than simple RGB.

  - Cons: Can confuse bare dry soil with concrete if spectral signatures are similar.

Lulu mall

Figure 1: Optical analysis showing new built-up areas (Cyan) overlaid on 2024 True Colour imagery.

B. SAR Analysis: Dual-Pol Log-Ratio

- Method: Applied a Log-Ratio change detection on VV and VH channels.

  - Log_Ratio = log10(Image_2024 / Image_2017)

- Why this method: Simple subtraction in SAR is prone to "speckle" noise. The ratio method effectively highlights relative changes in backscatter intensity. VV polarization was weighted heavily to detect "double-bounce" scattering typical of vertical walls and metal structures.

- Results: The method identified high-intensity changes corresponding to the mall's main structure. The vertical expansion of buildings was very clear in SAR.

  - Pros: Works day/night and through clouds; highly sensitive to man-made geometry (metal/concrete).

  - Cons: Susceptible to false alarms from temporary metal objects (vehicles, shipping containers).

Lulu mall

4. Geolocation Accuracy & Coregistration

- Input Accuracy: Sentinel-2 absolute geolocation accuracy is within 12.5m (2σ). Sentinel-1 is highly accurate due to orbital precision.

- Processing: All datasets were dynamically reprojected and coregistered to EPSG:32643 (UTM Zone 43N) using the 2024 Sentinel-2 pixel grid as the master reference. This ensures sub-pixel alignment between the optical and radar layers.

5. False Positives/Negatives

- False Positives:

  - Optical: Some riverbanks appeared as "buildup" due to high SWIR reflectance in the dry season.

  - SAR: High traffic density or temporary parking lots can mimic permanent structures due to corner reflection from vehicles.

- False Negatives:

  - Low-rise residential expansions under heavy tree canopy were missed by Optical (canopy obstruction) but partially captured by SAR.

6. Quality Checks & Edge Cases

- Edge Case: The "No-Data" boundaries (0 values) in SAR were handled by clamping inputs to a minimum of 0.001 to prevent logarithmic math errors.

- Cloud Artifacts: Although <10% cloud cover filters were used, thin cirrus clouds can slightly skew NDBI values. The SAR analysis provided a robust cross-check in these areas.

7. Recommendations

- Workflow: Integrate a "Water Mask" (NDWI) to automatically exclude river banks from the buildup detection, reducing false positives.

- UI Improvement: For a client-facing tool, add a "confidence slider" that allows the user to adjust the change threshold (currently fixed at 0.15 for Optical and 0.20 for SAR) in real-time.